Technology 1

August 28/29, 2008

Daily Assignment, AutoCAD Lines

BEFORE YOU START:  Create a new folder in your AUTOCAD folder, called “lastname_8-28” or “lastname_8-29”.  Save the following figures into this folder, call them figure1.dwg, figure2.dwg, etc.

FIGURE 1-  Use the LINE command to draw the following graphic design using absolute coordinates:

- LINE>Enter

- 1.5,5.75

- 3.25,5.75

- 3.25,4.5

- 5,6.375

- 3.25,8.25

- 3.25,7

- 1.5,7

- 2.125,6.375

- 1.5,5.75

- Enter to Cancel the Line Command

- Save drawing to your folder as figure1.dwg

FIGURE 2-  Use the LINE command to draw the following figure using absolute coordinates:

- 2,2

- 4,2

- 4,3.5

- 3,3.5

- 3,4

- 2,4

- 2,2

- ENTER to Cancel the Line Command

- LINE>Enter

- 2,2.75

- 4,2.75

- ENTER to Cancel the Line Command

- LINE>Enter

- 3,2

- 3,2.75

- ENTER

- Save drawing to your folder as figure2.dwg

FIGURE 3-  Use the LINE command to draw the sheet metal part using the following relative coordinates after the initial absolute coordinates:

- LINE>Enter

- 2, 2 (absolute)

- @3.25,0

- @0,1.875

- @-.875,0

- @0,.75

- @1.75,0

- @0,3.125

- @-.5,.5

- @-1.5,0

- @-.75,-.75

- @-.75,.75

- @-.25,0

- @0,-2

- @.25,0

- @0,-3.5

- @-.25,-.25

- @-.375,0

- Close

- Save the drawing to your folder as figure3.dwg

FIGURE 4-  Use the Line Command to draw the following figure using relative coordinates:

- LINE>Enter

- 6,2 (absolute)

- @2,0

- @0,2

- @-.75,0

- @0,-1.25

- @-1.25,0

- @0,-.75

- ENTER to Cancel Line Command.

- Save Drawing to your folder as figure4.dwg

FIGURE 5-  Use the LINE command to draw the latch plate using the following polar coordinates after the initial absolute coordinate:

- LINE> Enter

- 3,4 (absolute)

- @.5<0

- @.625<90

- @2.5<0

- @2.375<90

- @.75<180

- @1.5<270

- @2.25<180

- @.75<225

- @.375<270

- Close

- Save the drawing to your folder as figure5.dwg

FIGURE 6-  Use the LINE command to draw the trophy base using the following polar coordinates:

- LINE>Enter

- 3,4 (absolute)

- @2<45

- @2<315

- @1.5<270

- @2.8284<180

- @1.5<90

- Enter to cancel the Line Command

- Save the drawing to your folder as figure6.dwg

FIGURE 7-  Use the LINE command to draw the trophy base using the following combination of coordinates:

- 1.5,3 (absolute)

- @5,0 (relative)

- @1<90 (polar)

- @5<180 (polar)

- Close

- Press Enter or Space Bar

- 2.25,4 (absolute)

- @4.5<90 (polar)

- @1.75, 1.75 (relative)

- @1.75,-1.75 (relative)

- @4.5<270 (polar)

- Cancel

- Press Enter or Space Bar

- 2.75,4.75 (absolute)

- @2.5,0 (relative)

- @3.5<90 (polar)

- @2.5<180 (polar)

- Close

- Save the drawing to your folder as figure7.dwg

FIGURE 8-  Use the LINE command to draw the picture frame using the following combination of coordinates:

- Point 1:  1,1 (absolute)

- Point 2:  @6.5,0 (relative)

- Point 3:  @9<90 (polar)

- Point 4:  @-6.5,0 (relative)

- Close

- Press Enter or Space Bar

- Point 5:  1.5,1.5 (absolute)

- Point 6:  @5.5,0 (relative)

- Point 7:  @8<90 (polar)

- Point 8:  @5.5<180 (polar)

- Close

- Make sure that the Gray OBJ SNAP Button at the bottom of the screen is turned ON

- Press Enter or Space Bar

  - Click Point 1  (Points as defined in the problem above)

  - Click Point 5

  - Cancel

- Press Enter or Space Bar

  - Click Point 2

  - Click Point 6

  - Cancel

- Press Enter or Space Bar

  - Click Point 3

  - Click Point 7

  - Cancel

- Press Enter or Space Bar

  - Click Point 4

  - Click Point 8

  - Cancel

- Save the drawing to your folder as figure8.dwg

FIGURE 9-  Create a listing of all of the steps required to create the following figure.  (NOTE that the figure STARTS at point 2,2).  Please use all methods of Line Creation in this exercise:  Polar, Relative and Absolute.

Save the figure to your folder as Drawing 9.

Problems 10-14:  From the RED BOOK, replicate the following figures:  PAGE 31, Problems 1-2 and PAGE 32, Problems 3-4.

SUBMITTING YOUR WORK:  When you have completed all 13 problems, copy your entire “Lastname_8-28/29” folder to the M: Drive Drop Box for your class period.

GRADING:  I will grade 5 of the 13 problems.  If the graded figure is executed properly, you will receive 5 points for the problem.  If it is attempted, but executed incorrectly, you will receive 2.5 points.  If you have not completed that problem, you will receive 0 points.